| ### Lines |  |
| --- | --- |
|  | -continuous line - flow line |
|  | -dashed line - pilot, drain |
|  | -envelope - long and short dashes around two or more component symbols. |

### Circular. -large circle - pump, motor. -small circle - Measuring devices. -semi-circle - rotary actuator. ### Square. -one square - pressure control function -two or three adjacent squares - directional control. ### Diamond. -diamond - Fluid conditioner (filter, separator, lubricator, heat exchanger). ### Miscellaneous Symbols. -Spring. -Flow Restriction. ### Triangle. -solid - Direction of Hydraulic Fluid Flow. -open - Direction of Pnematic flow.. ## Pumps and Compressors. ### Fixed Displacement hydraulic pump. -unidirectional. -bidirectional. ### Variable displacement hydraulic pump. -unidirectional. -bidirectional. ### Compressor.. ## Motors. ### Fixed displacement hydraulic motor. -unidirectional. -bidirectional. ### Variable displacement hydraulic motor. -unidirectional. -bidirectional. ### Pneumatic motor. -unidirectional. -bidirectional. ### Rotary Actuator. - hydraulic. - pneumatic. ## Cylinders. ### Single acting cylinder. -returned by external force. -returned by spring or extended by spring force. ### Double acting cylinders. -single piston rod (fluid required to extend and retract). -double ended piston rod. ### Cylinders with cushions. - single fixed cushon. - double fixed cushion. - single adjustable cushion. - double adjustable cushion.. ## Directional Control Valves. ### Directional control valve (2 ports / 2 positions). -Normally closed directional control valve with 2 ports and 2 finite positions.. -Normally open directional control valve with 2 ports and 2 finite positions.. ### Directional control valve (3 ports / 2 positions). -Normally closed directional control valve with 3 ports and 2 finite positions.. -Normally open directional control valve with 3 ports and 2 finite positions.. ### Directional control valve (4 ports / 2 positions). -directional control valve with 4 ports and 2 finite postions. ### Directional control valve (4 ports / 3 positions). -directional control valve with 4 ports and 3 finite postions *-(center position can have various flow paths). ### Directional control valve (5 ports / 2 positions) Normally a pneumatic valve. -directional control valve with 5 ports and 2 finite postions. ### Directional control valve (5 ports / 3 positions) Normally a pneumatic valve. -directional control valve with 5 ports and 3 finite postions. ### Proportional directional control valve. ### Electro-hydraulic servo valve #### -The spool positions on these valves is variable allowing for variable flow conditions.. -single-stage direct operation unit which accepts an analog signal and provides a similar analog fluid power output. -two-stage with mechnical feedback indirect pilot operation unit which accepts an analog signal and provides a similar analog fluid power output.. ## Control Methods. ### Manual Control. -general symbol (without showing the control type). -pushbutton. -lever. -foot pedal. ### Mechanical Control. -plunger or tracer. -spring. -roller. -roller(one direction only). ### Electrical Control. -Solenoid (the one winding). ### Pilot Operation. -pneumatic. -hydraulic. ### Pilot operated two-stage valve. -Pneumatic: Sol first stage. -Pneumatic: Air pilot second stage. -Hydraulic: Sol first stage. -Hydraulic: Hyd pilot second stage.. ## Check valves, Shuttle valves, Rapid Exhaust valves. -check valve -free flow one direction, blocked flow in other direction. -pilot operated check valve, pilot to close. -pilot operated check valve, pilot to open. ### Shuttle valve. -to isolate one part of a system from an alternate part of circuit.. ### Rapid exhaust valve/Pneumatic. -installed close to an actuator for rapid movement of the actuator... ## Pressure Control Valves. ### Pressure Relief Valve(safety valve) normally closed. - line pressure is limited to the setting of the valve, secondary part is directed to tank.. ### Proportional Pressure Relief. - line pressure is limited to and proportional to an electronic signal. ### Sequence Valve. - when the line pressure reaches the setting of the valve, valve opens permitting flow to the secondary port. The pilot must be externally drained to tank.. ### Pressure Reducing valve; - pressure downstream of valve is limited to the setting of the valve.. ## Flow Control Valves. ### Throttle valve. -adjustable output flow. ### Flow Control valve. -with fixed output (variations in inlet pressure do not affect rate of flow). -with fixed output and relief port to reservoir with relief for excess flow (variations in inlet pressure do not affect rate of flow). -with variable output. -fixed orifice. -metered flow toward right free flow to left. -pressure compensated flow control fixed output flow regardless of load. -pressure and temperature compensated. -with variable output and relief port to reservoir. ### Flow dividing valve. -flow is divided equally to two outputs... ## Shut-Off Valve. -Simplified symbol.. ## Accumulators.. ## Filters, Water Traps, Lubricators and Miscellaneous Apparatus. ### Filter or Strainer. ### Water Trap. -with manual drain. -with automatic drained. ### Filter with water trap. -with manual drain. -automatic drain. ### Air Dryer. refrigerant, or chemical removal of water from compressed air line. ### Lubricator. -oil vapor is indected into air line. ### Conditioning unit. -compound symbol of filter, regulator, lubricator unit. -Simplified Symbol.. ## Heat Exchangers. <>; -air or water cooled unit designed to remove heat from oil returning to reservoir...